LAB 4 MINI GUIDE

Interrupt Driver APIs

Plain-English explanations and a one-button Linux 6.1 example

Purpose: Learn the minimum API set needed to convert a GPIO button from polling to interrupt-driven input with a 20 ms debounce delay and SIGIO notification.

This guide focuses on the APIs required by Overbug Lab 4. The final example uses the old integer-based GPIO interface because the course is pinned to the Raspberry Pi Linux 6.1 kernel.

# 1. The whole workflow

A button edge should not perform all the work. Each execution context performs one small job and passes the work to the next stage.

Driver workflow

Button changes electrical level

|

v

IRQ handler: restart the 20 ms debounce timer

|

v

Timer callback: read the stable GPIO state

|

v

Spinlock: safely store the new state

|

+----> Wake a process blocked in read()

|

+----> Send SIGIO to reader.c

Golden rule: The IRQ handler schedules work; the debounce timer checks the button; normal process context copies data to user space.

## Quick API map

| Job | Must-use APIs | Plain-English meaning |
| --- | --- | --- |
| Map GPIO | gpio_to_irq() | Find the interrupt number connected to a GPIO. |
| Handle edges | request_irq() free_irq() | Install the handler, then remove it safely. |
| Debounce | timer_setup() mod_timer() msecs_to_jiffies() del_timer_sync() | Wait until the electrical contacts stop bouncing. |
| Block read | DECLARE_WAIT_QUEUE_HEAD() wait_event_interruptible() wake_up_interruptible() | Let read() sleep until new data exists. |
| Protect state | DEFINE_SPINLOCK() spin_lock_irqsave() spin_unlock_irqrestore() | Prevent the timer and reader from changing shared data together. |
| Send SIGIO | fasync_helper() kill_fasync() | Tell reader.c that the device may have new data. |

# 2. Map a GPIO to an interrupt

## gpio_to_irq()

This function asks the GPIO subsystem for the IRQ number connected to one GPIO line. The return value is either a non-negative IRQ number or a negative error code.

Easy example

button_irq = gpio_to_irq(button_gpio);

if (button_irq < 0)

return button_irq;

Call it after requesting the GPIO and setting it as an input.

Keep the returned IRQ number so you can pass it to request_irq() and free_irq().

Do not assume that the IRQ number is the same as the GPIO number.

# 3. Register and release the interrupt

## request_irq()

This function connects an IRQ number to your handler. For a button, request both rising and falling edges so the driver sees both press and release.

Register one button IRQ

ret = request_irq(button_irq, button_irq_handler,

IRQF_TRIGGER_RISING | IRQF_TRIGGER_FALLING,

"simple_button", &button_irq);

if (ret)

return ret;

handler is the function the kernel calls when the edge occurs.

name appears in kernel diagnostics such as /proc/interrupts.

dev_id identifies this registration. Use the exact same pointer with free_irq().

## The IRQ handler

Keep the handler short. It should restart the debounce timer and return immediately.

Minimum handler

static irqreturn_t button_irq_handler(int irq, void *dev_id)

{

mod_timer(&debounce_timer,

jiffies + msecs_to_jiffies(20));

return IRQ_HANDLED;

}

## free_irq()

Call this while unloading the module and on initialization failure. It prevents the kernel from calling your handler again.

free_irq(button_irq, &button_irq);

Common mistake: Passing a different dev_id to free_irq(). It must match the pointer used by request_irq().

# 4. Debounce with a kernel timer

## timer_setup()

Initialize the timer once when the module loads. The callback runs later, after the requested delay.

timer_setup(&debounce_timer, debounce_callback, 0);

## msecs_to_jiffies()

Kernel timers use jiffies, not milliseconds. This function converts the 20 ms teaching value into the kernel's time unit.

delay = msecs_to_jiffies(20);

## mod_timer()

Start or restart the timer. Every bouncing edge pushes the expiration another 20 ms into the future.

mod_timer(&debounce_timer, jiffies + msecs_to_jiffies(20));

This restart behavior is the key to debounce: the callback runs 20 ms after the most recent edge, not 20 ms after the first edge.

## The timer callback

Read the GPIO here, compare it with the previous stable value, and publish only a real change.

Simplified callback

static void debounce_callback(struct timer_list *unused)

{

bool pressed = gpio_get_value(button_gpio) == 0;

/* Store and publish only if pressed changed. */

}

## del_timer_sync()

Stop the timer during cleanup and wait if its callback is already running. Call it after free_irq(), so no new IRQ can restart the timer.

Safe cleanup order

free_irq(button_irq, &button_irq);

del_timer_sync(&debounce_timer);

# 5. Let read() sleep until data is ready

## DECLARE_WAIT_QUEUE_HEAD()

Create a wait queue. A process that calls read() can sleep on this queue instead of repeatedly checking the driver.

static DECLARE_WAIT_QUEUE_HEAD(button_waitq);

## wait_event_interruptible()

Use this inside read(). It sleeps until data_ready becomes true. It may also return early if the process receives a signal, so always check its return value.

Block without polling

ret = wait_event_interruptible(button_waitq,

READ_ONCE(data_ready));

if (ret)

return ret;

## wake_up_interruptible()

After the timer callback stores a changed state, wake any process waiting for it.

wake_up_interruptible(&button_waitq);

Correct order: Store the state first. Set data_ready second. Wake the reader last. Otherwise, read() could wake before the new data is visible.

# 6. Protect the shared button state

## DEFINE_SPINLOCK()

Create a spinlock for the small piece of state shared by the timer callback and read().

static DEFINE_SPINLOCK(state_lock);

## spin_lock_irqsave() and spin_unlock_irqrestore()

Take the lock before reading or changing the shared state. The irqsave form is safe when the state may be used from interrupt-related contexts.

Publish state safely

unsigned long flags;

spin_lock_irqsave(&state_lock, flags);

stable_pressed = pressed;

data_ready = true;

spin_unlock_irqrestore(&state_lock, flags);

Keep the protected section short.

Do not call copy_to_user() while holding the spinlock.

Do not sleep while holding the spinlock.

# 7. Notify reader.c with SIGIO

## fasync_helper()

This helper records which user-space process asked for asynchronous notification. Connect it to the .fasync entry in file_operations.

Register or remove the SIGIO recipient

static int button_fasync(int fd, struct file *file, int on)

{

return fasync_helper(fd, file, on, &async_queue);

}

## kill_fasync()

Call this after a changed state has been stored. It sends SIGIO to the registered reader.

kill_fasync(&async_queue, SIGIO, POLL_IN);

SIGIO is only a doorbell. It says that the device may have data. reader.c must still call read() to receive the real button state.

## Remove the reader during close

Prevent signals after the reader closes

static int button_release(struct inode *inode, struct file *file)

{

button_fasync(-1, file, 0);

return 0;

}

Two notification paths: wake_up_interruptible() supports a blocking read(). kill_fasync() supports a SIGIO-based reader. Lab 4 uses both so the device remains usable in either style.

# 8. Complete one-button teaching example

This example demonstrates the required APIs with one active-low button on BCM GPIO 27. Supporting device-file and GPIO functions are included only so the example is complete.

Part 1 - headers and shared state

// SPDX-License-Identifier: GPL-2.0

#include <linux/fs.h>

#include <linux/gpio.h>

#include <linux/interrupt.h>

#include <linux/miscdevice.h>

#include <linux/module.h>

#include <linux/poll.h>

#include <linux/spinlock.h>

#include <linux/timer.h>

#include <linux/uaccess.h>

#include <linux/wait.h>

#define BUTTON_GPIO 27

#define DEBOUNCE_MS 20

static int button_irq;

static struct timer_list debounce_timer;

static DECLARE_WAIT_QUEUE_HEAD(button_waitq);

static DEFINE_SPINLOCK(state_lock);

static struct fasync_struct *async_queue;

static bool stable_pressed;

static bool data_ready;

Part 2 - IRQ and debounce timer

static void debounce_callback(struct timer_list *unused)

{

bool pressed = gpio_get_value(BUTTON_GPIO) == 0;

unsigned long flags;

bool changed = false;

spin_lock_irqsave(&state_lock, flags);

if (pressed != stable_pressed) {

stable_pressed = pressed;

data_ready = true;

changed = true;

}

spin_unlock_irqrestore(&state_lock, flags);

if (changed) {

wake_up_interruptible(&button_waitq);

kill_fasync(&async_queue, SIGIO, POLL_IN);

}

}

static irqreturn_t button_irq_handler(int irq, void *dev_id)

{

mod_timer(&debounce_timer,

jiffies + msecs_to_jiffies(DEBOUNCE_MS));

return IRQ_HANDLED;

}

Part 3 - read(), SIGIO registration, and device file

static ssize_t button_read(struct file *file, char __user *buf,

size_t count, loff_t *offset)

{

char result[2];

unsigned long flags;

bool pressed;

int ret;

if (count < sizeof(result))

return -EINVAL;

ret = wait_event_interruptible(button_waitq,

READ_ONCE(data_ready));

if (ret)

return ret;

spin_lock_irqsave(&state_lock, flags);

pressed = stable_pressed;

data_ready = false;

spin_unlock_irqrestore(&state_lock, flags);

result[0] = pressed ? '1' : '0';

result[1] = '\n';

if (copy_to_user(buf, result, sizeof(result)))

return -EFAULT;

return sizeof(result);

}

static int button_fasync(int fd, struct file *file, int on)

{

return fasync_helper(fd, file, on, &async_queue);

}

static int button_release(struct inode *inode, struct file *file)

{

button_fasync(-1, file, 0);

return 0;

}

static const struct file_operations button_fops = {

.owner = THIS_MODULE,

.read = button_read,

.fasync = button_fasync,

.release = button_release,

};

static struct miscdevice button_device = {

.minor = MISC_DYNAMIC_MINOR,

.name = "simple_button",

.fops = &button_fops,

.mode = 0444,

};

Part 4 - initialization and cleanup

static int __init button_init(void)

{

int ret;

timer_setup(&debounce_timer, debounce_callback, 0);

ret = gpio_request(BUTTON_GPIO, "simple_button");

if (ret)

return ret;

ret = gpio_direction_input(BUTTON_GPIO);

if (ret)

goto fail_gpio;

stable_pressed = gpio_get_value(BUTTON_GPIO) == 0;

button_irq = gpio_to_irq(BUTTON_GPIO);

if (button_irq < 0) {

ret = button_irq;

goto fail_gpio;

}

ret = request_irq(button_irq, button_irq_handler,

IRQF_TRIGGER_RISING | IRQF_TRIGGER_FALLING,

"simple_button", &button_irq);

if (ret)

goto fail_gpio;

ret = misc_register(&button_device);

if (ret)

goto fail_irq;

return 0;

fail_irq:

free_irq(button_irq, &button_irq);

del_timer_sync(&debounce_timer);

fail_gpio:

gpio_free(BUTTON_GPIO);

return ret;

}

static void __exit button_exit(void)

{

misc_deregister(&button_device);

free_irq(button_irq, &button_irq);

del_timer_sync(&debounce_timer);

gpio_free(BUTTON_GPIO);

}

module_init(button_init);

module_exit(button_exit);

MODULE_LICENSE("GPL");

# 9. What students should remember

gpio_to_irq() maps a GPIO number to an IRQ number.

request_irq() installs the handler; free_irq() removes it.

The IRQ handler does only one job: restart the debounce timer.

mod_timer() restarts the 20 ms delay after every bouncing edge.

The timer callback reads the stable GPIO value and publishes only changes.

A wait queue lets read() sleep instead of polling.

A spinlock protects the state shared by the timer callback and read().

fasync_helper() registers reader.c; kill_fasync() sends the SIGIO doorbell.

Store data before waking or signalling the reader.

During unload: stop new IRQ callbacks, stop the timer, then free the GPIO.

## Context safety checklist

| Context | Safe work in this lab | Do not do here |
| --- | --- | --- |
| IRQ handler | Restart the timer; return IRQ_HANDLED | Sleep, read all GPIOs, copy to user space |
| Timer callback | Read direct Pi GPIO; update small shared state; wake/notify | Sleep or perform long work |
| read() | Wait, take the spinlock briefly, copy data to user space | Hold the spinlock while sleeping or copying |
| Module exit | Deregister, free IRQ, stop timer synchronously, free GPIO | Free GPIO while callbacks can still run |

Final mental model: An interrupt is a doorbell, the debounce timer is a filter, the wait queue is a bed for read(), the spinlock is a one-person key, and SIGIO is a doorbell for reader.c.